Everdrive-MD hardware spec 1.1

09.07.2010

16 bit control registers:

| name | address | access |
| --- | --- | --- |
| SPI | 0xA13000 | R/W |
| CFG | 0xA13002 | W |
| STATE | 0xA13002 | R |
| VBLV | 0xA13004 | W |
| SRAMB | 0xA13006 | W |
| VER | 0xA13008 | R |
| ROM_MAP_PORT | 0xA1300A | W |

SPI:

This register used for access to SD/MMC card. SPI can be used in 8/16bit mode. Word or  byte mode can be select by CFG register. Also SPI have two speed  modes. In low speed mode, SPI req 128 CPU cycles, for read each byte, in high speed mode, SPI req 16 CPU cycles, for each byte. Low speed mode used only for init SD card. Speed mode select by CFG reg.

CFG:

| bit | Flag name | description |
| --- | --- | --- |
| 0 | #SS | Chip select for SPI |
| 1 | FULL_SPEED | SPI full speed |
| 2 | SPI16 | SPI 16 bit mode |
| 3 | GAME_MODE | Set cartridge in game mode. OS ROM memory and control registers not available in this mode |
| 4 | SMS_MODE | SMS game mode |
| 5 | HARD_RESET | Do hard reset. Necessary for enter in SMS game mode and recommend for enter in SMD game mode |
| 6 | RAM_MODE_1 | This backup ram option must be used for games which is more than 2 mbytes |
| 7 | RAM_ON | Enable backup ram usage |
| 8 | VBL_CATCH | Enable redirection for vblank interrupt vector. Vector will be redirected on address 0xff0000 + VBLV |
| 9 | MEGAKEY_ON | If true then console region will be changed on MEGAKEY_REGION_1 value |
| 10 | MEGAKEY_REGION_1 | 0 – NTSC, 1 - PAL |
| 11 | SSF2_MODE | 1 – SSF2 mode enable (firm v3+) |
| 12 | RAM_FS | 1 – save ram present as 1x32kb, 0- as 4x8kb (firm v3+) |
| 13 | CART | 1 – mega-cd bios loading. (firm v8+) |

* SSF2_MODE, SMS_MODE, RAM_MODE_1, SSF2_MODE will work only in game mode

* CART, VBL_CATCH will work in OS mode

* SMS_MODE + SSF2_MODE will turn on 32x mode (firm v4+) on firmware v3 it will set some unknown state (:

*save ram not present for 32x and SSF2 mode

STATE:

| bit | Flag name | description |
| --- | --- | --- |
| 0 | SPI ready | Set to 1 when read byte/word complete |
| 1 | EEPROM ready | Set to 1 when write/erase operation complete |
| 2 | key | Set to 1 if sms key pressed (firm v2+) |
| 3 | card | Set to 1 if SD cart pluged (firm v2+) |

VBLV:

Address for vblank interrupt redirection. ROM memory can not be read during erase/write operations, so vblank interrupt vector must be redirect in ram area. If CPU read data, from vblank vector address, and VBL_CATCH  set to 1, then on data bus lines set next value: 0xff0000 +  VBLV

SRAMB:

One of four sram bank select. Just write  value from 0 to 3

VER:

CPLD firmware version. 8bit number

ROM_MAP_PORT

Rom bank select. If  0 then flash memory area 0 – 0x400000 mapped on cartridge area, if 1 then flah memory area 0x400000 -  0x800000 mapped on cartridge area.

Memory map in OS mode: (firm 1 - 2)

0x000000 – 0x040000: OS ROM area

0x040000 – 0x100000: module ROM

0x100000 – 0x200000: reserved area

0x200000 – 0x400000: unused (if RAM_ON 1)

0x200000 – 0x400000: mirrored 8kb of  backup ram (if RAM_ON 1)

0x400000 – 0x800000: game ROM

0xA13000 - 0xA1300A: control registers

Memory map in OS mode: (firm 3+)

0x000000 – 0x040000: OS ROM area

0x040000 – 0x100000: module ROM area

0x100000 – 0x200000: reserved area

0x200000 – 0x400000: mirrored 8kb of  backup ram (if RAM_ON 1)

0x200000 – 0x400000: game ROM (start of  SSF2 rom if RAM_ON 0)

0x000000 – 0x400000: game ROM  (if in ROM_MAP_PORT  written 1)

0xA13000 - 0xA1300A: control registers

Memory map in SMD GAME mode

0x000000 – 0x400000: game ROM area (if RAM_ON 0)

0x000000 – 0x200000: game ROM area (if RAM_ON 1)

0x200000 – 0x400000: 8kb of  backup ram (if RAM_ON 1)

OS ROM memory map:

0x000000 - 0x010000: bootloader

0x010000 - 0x020000: OS

0x020000 - 0x030000: reserve OS copy

0x030000 - 0x040000: settings data

*reserved OS copy can be run if A+B+C+UP keys will be pressed before system power on

*new carts use A+B+C combination for reserved OS copy loading

Backup memory:

Cart have 32kb of  Ferroelectric Nonvolatile RAM in four 8kb banks. This memory not need battery supply for keep data. Cart can keep save data for four different games in one time.